# Worcester Polytechnic Institute  Department of Electrical and Computer Engineering

## ECE2311  Continuous-Time Signal and System Analysis  Term B’17

# Homework 2:  Due Tuesday, 31 October 2017 (3:00 P.M.)

Write your name and ECE box at the top of each page.

General Reminders on Homework Assignments:

- Always complete the reading assignments before attempting the homework problems.

- Show all of your work.  Use written English, where applicable, to provide a log or your steps in solving a problem.  (For numerical homework problems, the writing can be brief.)

- A solution that requires physical units is incorrect unless the units are listed as part of the result.

- Get in the habit of underlining, circling or boxing your result.

- Always write neatly.  Communication skills are essential in engineering and science.  “If you didn’t write it, you didn’t do it!”

_____________________________________________________________________________________

- Basic Signals:

  - By hand (without using a graphing calculator, etc.), sketch and label on the same axes: , ,  and .  Express t in seconds.

  - Sketch and label: , where  is the unit step function and  is the unit ramp function.

  - Use MATLAB’s numerical software (not its symbolic software) to plot  over the range t = [–20, 20] seconds.  Be sure (i) to use sufficient resolution to accurately show the waveform shape (the time values –20, –19, –18, …, 20 will not be adequate), and (ii) to generate the correct value at t = 0.  Note that if you allow MATLAB to compute the value when t = 0, the division by 0 will result in the NaN value (NaN = “Not a Number”).  The actual value at t = 0 can be found analytically by application of L’Hôpital’s rule, with that value equaling x(0) = 1.  Find a way to force MATLAB to plot this value at t = 0.  Hand-in printouts of your plot and your MATLAB code.  Be sure that all axes are properly labeled by using the MATLAB xlabel() and ylabel() commands.

  - Determine the signal power in: .

  - Determine the signal energy in: .

- The Impulse:  Let  be the impulse function.

  - Simplify: .

  - Simplify: .

  - Evaluate: .

  - Evaluate: .

- Classification of Signals:

  - Determine if each of the following signals is periodic.  If so, list its frequency in Hertz.

    - .

    - .

    - .

  - For each below, let y(t) be the system output and x(t) be the system input.  For each, determine if the system is static vs. dynamic and causal vs. non-causal.

    - .

    - .

    - .

    - .

  - Determine and sketch the even and odd parts of: , where  is the step function.

  - Determine and sketch the even and odd parts of: .